# Installation Instructions for OTCD in Linux

## General Instructions for Linux Beginners

- Some OpenTurbineCoDe dependencies work on Windows, but others are limited to Linux, so it makes sense to just use OpenTurbineCoDe in Linux. If you have a Windows computer, you will need the Windows Subsystem for Linux.

- To get the Windows Subsystem for Linux, open the Windows Command Prompt as an administrator and type wsl --install. Once it finishes installing, restart your computer.

  - Once the computer restarts, it should automatically install Ubuntu. It may prompt you to choose a username and password.

- Pip, make, cmake, and the gcc suite of code compilers are important tools for compiling and installing programs.

  - To install pip in the Ubuntu command prompt, type sudo apt update followed by sudo apt install python3-pip.

  - To install make and the compilers, type sudo apt install gcc build-essential make -y.

  - To install cmake, type sudo apt-get install cmake.

- To navigate the file system on Ubuntu, type cd and then a folder name to go one level deeper into your computer’s file system, or type cd .. to move one level backwards.

  - The default directory when you open Ubuntu is /home/username. If you wish to navigate into the standard Windows file system whose root directory is C:\, that is renamed to /mnt/c in Ubuntu.

  - To create a directory within the current folder, type mkdir folder_name.

- There are three good ways of downloading programs off Github.

  - Git clone: To download the most recently updated version of the program, click the green “Code” button on the Github site, then copy the https://github.com/program_name.git link. Navigate in Ubuntu to the folder where you want to download and type: git clone https://github.com/program_name.git. This will download the program.

  - Download zip: If using git clone doesn’t work, you can click the green “Code” button on the Github site and then “Download ZIP.” This will download a zip file of the program, which you can extract to the folder of your choice.

  - From release: If you want to download a past release of the program (likely to be more stable), click “Releases” on the Github page.

    - Scroll to the desired release, and under “Assets,” right click on the “Source Code (tar.gz)” file and copy the link.

    - Navigate in Ubuntu to the desired destination folder, then type wget and paste the link.

    - Once it downloads, extract the files with tar -xvzf file_name.tar.gz.

- Your computer can only find programs if the location of the program is specified or if the folder containing the program is saved in the computer’s path environment variable. To save a folder to the path in WSL, open the .bashrc file located in the /home/username directory. At the bottom of .bashrc, add the line export PATH=$PATH:/path/to/folder.

  - Python programs can find modules if the folder containing the module is saved in the PYTHONPATH environment variable. To save a folder to PYTHONPATH in WSL, open the .bashrc file located in the /home/username directory. At the bottom of .bashrc, add the line export PYTHONPATH=$PYTHONPATH:/path/to/folder.

## OpenTurbineCoDe Installation

- To run OpenTurbineCoDe, the following dependencies are required: Numpy, jsonschema, Matplotlib, scp, pyqtgraph, and PyQt5. The first five of those can be installed with pip; type in Ubuntu: pip install Numpy, pip install jsonschema, etc.

  - For PyQt5, instead of using pip, install it with sudo: sudo apt-get install python3-pyqt5.

- Download the OpenTurbineCoDe code from Github: https://github.com/OpenTurbineCoDe/OpenTurbineCoDe

- To install OpenTurbineCoDe, navigate in Ubuntu into the OpenTurbineCoDe folder and type pip install -e.

## OpenFAST/AeroDyn Driver/BeamDyn Driver

- OpenFAST documentation can be found here: https://openfast.readthedocs.io/en/main/source/install/index.html

- Download the OpenFAST 2.4.0 release from Github: https://github.com/OpenFAST/openfast/releases/tag/v2.4.0

- Download dependencies with sudo apt install git cmake libblas-dev liblapack-dev gfortran-10 g++

- Within the openfast-2.4.0 directory, create a build folder with mkdir build and then move into it.

- Within the openfast-2.4.0/build folder, run cmake ...

- To create executable files for OpenFAST, the standalone AeroDyn Driver (used in the Aerodynamics module) and standalone BeamDyn Driver (used in the Structures module), run make openfast aerodyn_driver beamdyn_driver. This may take several minutes.

- To test whether the compilation worked, run ./glue-codes/openfast/openfast -v, or ./modules/aerodyn/aerodyn_driver -v, or ./modules/beamdyn/beamdyn_driver -v.

- For OpenFAST, AeroDyn, and BeamDyn to run in OpenTurbineCoDe, they must be added to the config file. To do this, open the file OpenTurbineCoDe > openturbinecode > config.json , and next to where it says “path_to_aerodyn” paste the address of the aerodyn_driver file. If it is not the last entry in the section, the address should be followed by a comma (e.g. “/home/username/openfast-2.4.0/build/modules/aerodyn/aerodyn_driver”,). Repeat this process for openfast and for beamdyn_driver.

- If you want to test OpenFAST with reference wind turbines, follow this procedure (if you only intend to use OpenFAST in concert with OpenTurbineCoDe, this is unnecessary):

  - Go to https://github.com/OpenFAST/r-test/tree/v2.4.0

  - Click on the green Code button and then click Download ZIP

  - Extract the contents to your openfast folder.

  - If you want to run tests with the 5MW reference turbine, navigate to r-test-2.4.0 > glue-codes > openfast > 5MW_Baseline > ServoData. There should be three folders there: DISCON, DISCON_ITI, and DISCON_OC3. For the DISCON folder, create a build folder inside of it, then run cmake .., run make, and copy the .dll file that’s created into the ServoData directory. Repeat with DISCON_ITI and DISCON_OC3.

  - To run a test on for instance, the 5MW_OC3Mnpl_DLL_WTurb_WavesIrr turbine, navigate to r-test-2.4.0 > glue-codes > openfast > 5MW_OC3Mnpl_DLL_WTurb_WavesIrr and then type the path to the openfast executable, then a space, and then the fst file. In this example, type: /home/tanig/wind/openfast-2.4.0/build/glue-codes/openfast/openfast 5MW_OC3Mnpl_DLL_WTurb_WavesIrr.fst

## PyFAST

- PyFAST is the python compatibility module of OpenFAST. It is required in order to run the BeamDyn structures module of OpenTurbineCoDe, and source code can be found here: https://github.com/OpenFAST/python-toolbox.

- To install PyFAST, download the code from https://github.com/OpenFAST/python-toolbox to the folder of your choice.

- Navigate into the python-toolbox folder and type pip install -e., which should install the program.

## OpenCascade

- OpenCascade is a CAD program required by BB3D. BB3D requires an OpenCascade version between 7.0.0 and 7.4.0. To download OpenCascade, go to https://dev.opencascade.org/release/previous and click the yellow “Download OpenCascade Technology” button. If prompted, make an account on their website.

- Move the opencascade-versionnumber.tgz to the folder of your choice and use tar -xvzf opencascade-versionnumber.tgz to unzip it.

- Install OpenCascade’s many dependencies with sudo apt-get install tcllib tklib tcl-dev tk-dev libfreetype-dev libxt-dev libxmu-dev libxi-dev libgl1-mesa-dev libglu1-mesa-dev libfreeimage-dev libtbb-dev.

- Within the opencascade folder, create a build directory, then run cmake .. and make. This may take a long time.

- If you want to visualize IGES CAD files in OpenCascade, environment variables must be configured.

  - In env.sh in the root opencascade directory, change the line export PATH="${CSF_OCCTBinPath}:${PATH}" to include the path to the directory where the DRAWEXE executable is, e.g. export PATH="$CASROOT/build/lin64/gcc/bin:${PATH}".

  - Also in env.sh, change the line export LD_LIBRARY_PATH="${CSF_OCCTLibPath}:${LD_LIBRARY_PATH}" to include the path to the OpenCascade library folder, e.g. export LD_LIBRARY_PATH="$CASROOT/build/lin64/gcc/lib:${LD_LIBRARY_PATH}".

  - In draw.sh in the root opencascade directory, change the line DRAWEXE to include the OpenCascade version number, e.g. DRAWEXE-7.3.0.

## BB3D

- Download the BB3D repository from https://github.com/smbarr/BB3D.

- Specify the paths to the OpenCascade include folder and library. To do this, open the CMakeLists.txt file from the BB3D folder.

  - Change the set (OCE_INCLUDE_DIR "" CACHE PATH "Path to Open CASCADE include directory.") line so it reads set (OCE_INCLUDE_DIR /path/to/opencascade-7.6.0/inc).

  - Then change the set (OCE_LIB_DIR "" CACHE PATH "Path to Open CASCADE library directory.") file so it reads set (OCE_LIB_DIR /path/to/opencascade-7.6.0/build/lin64/gcc/lib).

- Navigate into the BB3D folder and create a new subfolder called “bin.”

- In the bin folder, type cmake .. and then make. This should generate an executable file named make_blade.

- Move the make_blade file from the bin folder to the data folder within BB3D. This will enable it to see its input files.

- To run BB3D by itself, navigate to the data folder and type ./make_blade.

  - If there is a “Permission denied” error, check the permissions of the make_blade executable by typing ls -l make_blade. If the output line does not contain the letter x, you do not have permission to execute the program. To give yourself executing power, type chmod u+x make_blade.

- Once you have generated a blade.igs file, you can view it in OpenCascade by running ./draw.sh from the root opencascade folder.

  - Then type pload DATAEXCHANGE

  - Then type ReadIges D1 /path/to/blade.igs

  - Then type XShow D1

  - Then type vfit

- To run BB3D from OpenTurbineCoDe, add the address to make_blade to the config.json file next to “path_to_BB3D.”

## PGL

- Source code can be found here: https://gitlab.windenergy.dtu.dk/frza/PGL.

- To download the code: Navigate in Ubuntu to the folder of your choice and type git clone https://gitlab.windenergy.dtu.dk/frza/PGL.git.

- To install PGL, navigate into the PGL folder and type python3 setup.py develop --prefix=path/to/installation/directory. This should install PGL.

- If the installation message says so, you may need to add a directory or two to the PYTHONPATH in Linux. To do this from WSL, go to the /home/username directory, open the .bashrc file, and add the following line to the bottom: export PYTHONPATH=“/path/to/directory”. Then save the file and close and restart Ubuntu.

  - To check whether it worked, type echo ${PYTHONPATH} in Ubuntu.

  - Now repeat step 3 until there are no installation errors.

## Salome

- The Salome module requires an input file from BB3D, so make sure you have the BB3D module installed before using Salome.

- To download Salome, go to https://www.salome-platform.org/?page_id=15 and download the Ubuntu software for your operating system. Unzip this with tar -xvzf.

- Salome has many software dependencies. To check whether you have all of them downloaded, navigate into the root SALOME folder and run ./sat/sat config SALOME-<version_number>-native --check system

  - Install the missing dependencies with sudo apt-get install.

  - If you get errors trying to install dependencies, you may need to update apt-get by typing sudo apt-get update.

- To run Salome, run ./salome from the root SALOME folder.

- Add the path of the salome executable to config.json next to “path_to_Salome.”

## TACS

- TACS documentation can be found at https://smdogroup.github.io/tacs/install.html

- Clone the TACS repository (https://github.com/smdogroup/tacs) into the folder of your choice by typing git clone https://github.com/smdogroup/tacs.git.

- TACS dependencies are intended to be installed in the folder tacs > extern:

  - To install GKlib (https://github.com/KarypisLab/GKlib), navigate to tacs > extern, then type git clone https://github.com/KarypisLab/GKlib.git.

    - Then navigate into the GKlib folder and type make config.

    - Then type make.

    - Then type make install.

  - To install METIS (https://github.com/KarypisLab/METIS), navigate to tacs > extern, then type git clone https://github.com/KarypisLab/METIS.git.

    - Then navigate into the METIS folder and type make config.

    - Then type make install.

  - To install MPI, download the most recent version of the tar.gz zipped file from https://www.open-mpi.org/software/ompi/.

    - To extract the contents of the file, type tar xvzf path/to/openmpi-version_number.tar.gz -C path/to/tacs/extern. (Note that in Linux, the standard Windows C:\ root directory is replaced with /mnt/c/).

    - Navigate into the openmpi folder, create a build directory with mkdir build, and then navigate into the build folder.

    - Type ../configure --prefix=/home/username/local to install the OpenMPI libraries in the same place as the GKLib and METIS ones. This may take a few minutes.

    - Type make all install. This may take a few minutes.

  - To install LAPACK, download the most recent version of the tar.gz zipped file from https://netlib.org/lapack/.

    - To extract the contents of the file, type tar xvzf path/to/lapack-version_number.tar.gz -C path/to/tacs/extern.

    - Install the gfortran compiler with sudo apt install gfortran.

    - Within the lapack folder, find the make.inc.example file and rename it make.inc

    - Open make.inc and, at the bottom, where it defines the build locations of BLASLIB, CBLASLIB, etc., change the root directories of all the libraries from $(TOPSRCDIR) to ${HOME}/local/lib. This will install the LAPACK libraries in the same place as the libraries of the other dependencies.

    - Navigate into the lapack folder and type make. This may take a few minutes.

- In the tacs folder, duplicate the Makefile.in.info file. Then rename one of the copies “Makefile.in”

  - Open the newly renamed Makefile.in and change the definition of TACS_DIR to be the location of the tacs folder.

  - Change the definition of CXX to be your C++ compiler, likely g++.

  - Change the definition of LAPACK_LIBS to -llapack -lpthread -lrefblas

  - Change METIS_INCLUDE  to be the location of metis.h, which should be -I${HOME}/local/include.

  - Change METIS_LIB to be the location of libmetis.a, which should be -L${HOME}/local/lib

- Then, in the tacs folder, type make.

- In order to create the interface for TACS, you must have Python pip installed on Ubuntu. To do this, type sudo apt update followed by sudo apt install python3-pip.

  - Now install MPI for Python by typing sudo apt-get install libopenmpi-dev and then pip install mpi4py.

  - Install Cython by typing pip install Cython.

- Then, in the tacs folder, type make interface.

## ADFlow

- Install OpenMPI, PETSc, CGNS, Numpy, mpi4py, and petsc4py (in that order) with instructions from https://mdolab-mach-aero.readthedocs-hosted.com/en/latest/installInstructions/install3rdPartyPackages.html

- If you are using Ubuntu with GCC compilers, navigate into the adflow folder and type cp config/defaults/config.LINUX_GFORTRAN.mk config/config.mk to copy the proper config.mk file into the config directory.

- Ubuntu calls python with the command python3. To tell this to ADFlow, open the config.mk file you just copied and ensure that the line beginning with PYTHON = reads PYTHON = python3.

- Type make.

  - If it spits out an error, you may need to open the Makefile in src > build and change the following lines:

    - python ../f2py/checkPyfForUpperCase.py

    - python ../f2py/pyf_preprocessor.py real ../f2py/adflow.pyf

  - To:

    - $(PYTHON) ../f2py/checkPyfForUpperCase.py

    - $(PYTHON) ../f2py/pyf_preprocessor.py real ../f2py/adflow.pyf

  - If you get an error in the make, type make clean before you try to make again.

- Download MultiPoint from https://github.com/mdolab/multipoint

  - Navigate into the multipoint folder and type pip install ..

## PyGeo

- First download PySpline from here: https://github.com/mdolab/pyspline/. Documentation is here: https://mdolab-pyspline.readthedocs-hosted.com/en/latest/building.html

- Copy the proper config.mk file into the config folder. If using GCC compilers on Linux, do this by navigating into the pyspline folder and typing cp config/defaults/config.LINUX_GFORTRAN.mk config/config.mk.

- Open the file pyspline > config > config.mk and ensure that the PYTHON variable equals python3.

- Then, in the pyspline root folder, type make and then pip install ..

- Download PyGeo from here: https://github.com/mdolab/pygeo

  - Don’t use release 1.12 — it’s buggy.

- Navigate into pygeo folder and type pip install .. That should install PyGeo (https://mdolab-pygeo.readthedocs-hosted.com/en/latest/install.html).

## WEIS

- OpenTurbineCoDe supports only a very specific version of WEIS. To download the WEIS version from January 8, 2021, go to https://github.com/WISDEM/WEIS/tree/06765cec83d7e2e5d9b86e31829b1e66b9decbdc, click on the green “Code” button, and Download ZIP. Extract the files to the folder of your choice.

- Navigate inside the WEIS folder and type python3 setup.py develop --prefix=path/to/installation/directory. The installation directory should be saved to the PYTHONPATH.

- Now build the libdiscon library for the ROSCO controller. To do this, first create a build directory in WEIS > ROSCO. Navigate inside this build folder and type cmake ... Then type make to create libdiscon.so.

  - Copy the libdiscon.so file and paste it into the OpenTurbineCoDe > openturbinecode > controls folder. This will enable OpenTurbineCoDe to find it.

# Installation Instructions for OTCD in Windows

## OpenTurbineCoDe Installation

- Install Python at Download Python | Python.org

- Install Git at Git for Windows . This will help to download programs off Github.

- Open the Command Prompt on your computer.

- To run OpenTurbineCoDe, the following code packages are required: Numpy, jsonschema, Matplotlib, scp, PyQt5, and pyqtgraph. They can be installed with pip -- type in the command prompt: pip install Numpy, pip install jsonschema, etc.

  - Note: If using a Linux distribution, instead of using pip for PyQt5, install it with sudo apt-get install python3-pyqt5.

- To download OpenTurbineCoDe, navigate in the Command Prompt to the folder where you would like to download it and type: git clone https://github.com/OpenTurbineCoDe/OpenTurbineCoDe.git . Then press Enter.

  - To navigate the file system on Windows Command Prompt, type cd and then a folder name to go one level deeper into your computer’s file system, or type cd.. to move one level backwards.

- To install OpenTurbineCoDe, navigate in the Command Prompt into the OpenTurbineCoDe folder. Then type pip3 install -e.

## OpenFAST

- OpenTurbineCoDe uses OpenFAST version 2.4. To download this:

  - Go to https://github.com/OpenFAST/openfast/releases/tag/v2.4.0

  - Under “Assets,” click on openfast_v2.4.0_binaries.zip to download the program. Then extract it to the folder of your choice.

- Open your OpenTurbineCoDe folder, navigate to openturbinecode > config.json , and next to where it says “path_to_openfast” paste the address of the openfast_24.exe file, with double backslashes between each layer and followed by a comma (e.g. "C:\\Users\\tanig\\Wind\\openfast_2.4\\openfast_24.exe",)

- OPTIONAL: If you want to test OpenFAST with reference wind turbines, you may want to add the openfast folder to your computer’s environment path:

  - Open the Settings app.

  - Within System settings, click About

  - Under Device Specifications, click Advanced System Settings

  - Click Environment Variables

  - Under System Variables, scroll down and click on “Path.”

  - Click the “Edit” button on the right-hand side.

  - Click “New,” and then copy and paste the location of your openfast folder (e.g. C:\Users\tanig\Wind\openfast_2.4).

- OPTIONAL: If you want to test OpenFAST with reference wind turbines, follow this procedure (if you only intend to use OpenFAST in concert with OpenTurbineCoDe, this is unnecessary):

  - Go to https://github.com/OpenFAST/r-test/tree/v2.4.0

  - Click on the green Code button and then click Download ZIP

  - Extract the contents to your openfast folder.

  - If you want to run tests with the 5MW reference turbine, navigate from your openfast root folder to DISCON_DLLS > 64bit. Then copy all three dll files, navigate to r-test-2.4.0 > glue-codes > openfast > 5MW_Baseline > ServoData. Paste the three files there.

  - To run a test on for instance, the 5MW_OC3Mnpl_DLL_WTurb_WavesIrr turbine, navigate to r-test-2.4.0 > glue-codes > openfast > 5MW_OC3Mnpl_DLL_WTurb_WavesIrr and then type the path to the openfast executable followed by the fst file. In this example, if you have the openfast folder added to the path, type: openfast_x64.exe 5MW_OC3Mnpl_DLL_WTurb_WavesIrr.fst

## Standalone AeroDyn Driver/BeamDyn Driver

- OpenTurbineCoDe uses the AeroDyn Driver and BeamDyn Driver modules of OpenFAST, which must be manually compiled from the source code. The steps for installing both of these modules are nearly identical. Documentation for doing this is located here: 2. Installing OpenFAST — OpenFAST v3.1.0 documentation.

- First, ensure that you have Microsoft Visual Studio installed Download Visual Studio Tools - Install Free for Windows, Mac, Linux (microsoft.com). As of July 2022, the Intel Fortran Compiler (see next step) is not compatible with the latest version of Visual Studio, so you may need to install the 2019 version of Visual Studio instead. This can be found here: Visual Studio Older Downloads - 2019, 2017, 2015 & Previous Versions (microsoft.com)

- Once Visual Studio is installed, download the Intel Fortran Compiler from https://www.intel.com/content/www/us/en/developer/articles/tool/oneapi-standalone-components.html#fortran . Follow the installation instructions to integrate the compiler with Visual Studio.

- Now you are ready to download the AeroDyn/BeamDyn Driver 2.4 source code. Go to https://github.com/OpenFAST/openfast/tree/v2.4.0 , click on the green “Code” button, and click Download ZIP. Extract the contents of the ZIP to the folder of your choice.

- For AeroDyn: Within the folder you just extracted, navigate to vs-build > Aerodyn and open the AeroDyn_Driver.sln file in Visual Studio.

  - For BeamDyn: Navigate to vs-build > Beamdyn and open the BeamDyn-w-registry.sln file in Visual Studio.

- Within Visual Studio, at the bar on top of the page, make sure that the Solution Configuration box says “Release” (not “Debug”) and that the Solution Platform box next to it says x64 (not Win32). Then click Build > Build Solution to compile the AeroDyn/BeamDyn Driver. Once it finishes compiling, it should create an AeroDyn_Driver_x64.exe or BeamDyn_Driver_x64.exe file under openfast > build > bin.

- Open your OpenTurbineCoDe folder, navigate to openturbinecode > config.json , and next to where it says “path_to_aerodyn” paste the address of the AeroDyn_Driver_x64.exe file, with double backslashes between each layer and followed by a comma (e.g. "C:\\Users\\tanig\\Wind\\openfast_2.4\\source_code\\build\\bin\\AeroDyn_Driver_x64.exe",). For BeamDyn, paste the address of the BeamDyn exe file next to path_to_beamdyn.

## PyFAST

- PyFAST is the python compatibility module of OpenFAST. It is required in order to run the BeamDyn structures module of OpenTurbineCoDe, and source code can be found here: https://github.com/OpenFAST/python-toolbox.

- To install PyFAST, download the code with git clone https://github.com/OpenFAST/python-toolbox.git to the folder of your choice.

- Navigate into the python-toolbox folder and type pip install -e., which should install the program.

## OpenCascade and BB3D

- Ensure that you have CMake installed from https://cmake.org/download/ . This will be used to generate a BB3D solution file before it can be compiled.

- BB3D requires a CAD software called OpenCascade. The best version for this purpose is OpenCascade 7.3.0. Download the OpenCascade 7.3.0 binaries from https://dev.opencascade.org/release/previous . It may require you to make an account in order to download. Open the installer and follow its installation instructions.

- Clone the BB3D repository by opening a command prompt and typing: git clone https://github.com/smbarr/BB3D.git

- To assemble BB3D, first navigate into the BB3D folder and create a new subfolder called “bin.” This is where you will build the BB3D binaries.

- Next, open the CMake graphical interface by navigating to CMake > bin and running cmake-gui.exe

  - For the “Where is the source code” box, click the “Browse Source” button and select the BB3D folder.

  - For the “Where to build the binaries” box, click the “Browse Build” button and select the BB3D > bin folder you just created.

  - Click the Configure button on the bottom. When prompted, select your version of Microsoft Visual Studio and then click “Finish.”

  - This should spit out a few red lines. Next to OCE_INCLUDE_DIR insert the address of the OpenCASCADE > opencascade > inc folder, and next to OCE_LIB_DIR insert the address of the OpenCASCADE > opencascade > win64 > vc > lib folder. Then click “Configure” again. Then click “Generate” and then “Open Project.” This should open Visual Studio.

- In Visual Studio, change the boxes at the top of the page to say “Release” and “x64” and then click on the Build menu > Build Solution.

- The BB3D executable file requires airfoil data files in order to function. Therefore, copy the make_blade.exe file from BB3D > bin > Release and paste it in BB3D > data.

- The BB3D executable file also requires some libraries from OpenCascade. Therefore, copy the contents of OpenCASCADE > opencascade > win64 > vc > bin and paste them in BB3D > data.

- You can now run the make_blade.exe program from the command line. When run, this program takes the blade data from data.dat and generates an IGES CAD file.

- You can view this CAD file in OpenCascade by, in the command line, running draw.bat from OpenCASCADE > opencascade.

  - Then type pload DATAEXCHANGE

  - Then type ReadIges D1 [path to blade.igs]

  - Then type XShow D1

  - Then type vfit

- To run BB3D from OpenTurbineCoDe, add the address to make_blade.exe (with double backslashes between levels) to the config.json file next to “path_to_BB3D.”

## PGL

PGL cannot be compiled on Windows. See its installation instructions for Linux.

## Salome

- The Salome module requires an input file from BB3D, so make sure you have the BB3D module installed before using Salome.

- To download Salome, go to https://www.salome-platform.org/?page_id=15 and download the correct software for your operating system. Follow its installation instructions.

- Add the path of run_SALOME.exe to config.json next to “path_to_Salome.”

## TACS

TACS cannot be installed in Windows. See the TACS installation instructions for Linux.

## ADFlow

ADflow cannot be installed in Windows. See the ADflow installation instructions for Linux.