# Documentation for Use of OpenTurbineCoDe

# 1. General Information

OpenTurbineCode (OTCD) is an open-source wind turbine modeling software. It serves as a wrapper around several other modeling programs, most notably OpenFAST, in order to give the user many different independent modules and many different choices of fidelity. Each module of OTCD has its own dependent programs and will run perfectly well even if another module’s dependencies are not installed.

OTCD can be run from a GUI in addition to directly from the command line. For documentation on installing OTCD or any of its dependencies, see Installation Instructions. To run the GUI, navigate in the command prompt to the OpenTurbineCoDe > openturbinecode folder and then type main.py --GUI . If running a Linux distribution, you may need to type python3 main.py --GUI .

# 2. Main Options Module

Dependencies: Python ≥ 3.6, Numpy, jsonschema, Matplotlib, scp, PyQt5, pyqtgraph

As of August 2022, OpenTurbineCoDe only supports modeling of a reference turbine, the 10-megawatt turbine designed by the Technical University of Denmark (DTU10MW). The specifications for this reference turbine are pre-saved in OpenTurbineCoDe (OTCD) in the following file: OpenTurbineCoDe > models > DTU_10MW > Madsen2019 > Madsen2019_10.yaml . To load this information in the OTCD GUI, click the “Preload turbine” button.

Functionality for the “Set working folder” box has not yet been developed as of August 2022, so that can be ignored for now.

If you pre-loaded the DTU10MW turbine, the Parameters tab should automatically fill in with the reference turbine’s power, number of blades, rotor diameter, etc.

# 3. Geometry Module

If you pre-loaded the DTU10MW turbine, the left side of the GUI should be filled with blade parameters: position along the blade, chord length, twist, and airfoil number. If you wish to choose a different blade to model, use the “Load Blade” box in the top left. It requires an AeroDyn blade file.

The airfoil numbers in the blade parameters table correspond to different airfoil prototypes that are pre-saved in an airfoil library in OTCD. The default settings of which number corresponds to which airfoil type come from the DTU10MW file, but they can be changed with the drop-down menu.

## 3.1 AeroDyn

Dependencies: None

This module creates a text-based input file for blade geometry. To write an AeroDyn blade file, select “AeroDyn blade file” from the drop-down menu and then click “Generate geometry/mesh.” This will create an AeroDyn blade file in OpenTurbineCoDe > models > DTU_10MW > Madsen2019 > Aerodyn. An additional copy of the blade file goes to models > DTU_10MW > Madsen2019 > OpenFAST.

## 3.2 TurbinesFOAM

Dependencies: None

This module creates a text-based input file for blade geometry. To write a TurbinesFOAM blade file, select “turbinesFoam file” from the drop-down menu and then click “Generate geometry/mesh.” This will create a TurbinesFOAM blade file and an airfoil file in OpenTurbineCoDe > models > DTU_10MW > Madsen2019 > turbineFOAM.

## 3.3 BB3D

Dependencies: OpenCascade, BB3D

### 3.3.1 Inputs

This module creates a CAD model for blade geometry. Select “BB3D” from the drop-down menu. If you set the location of the BB3D executable file in config.json, the “Set BB3D exe” box should be pre-filled. If not, fill it in with the location of your BB3D executable.

If you wish to change the number and/or location of spars along the blade, update the “set spar locations” box.

BB3D builds a blade model using a specific number of lofts which must be pre-set beforehand. Choose the number of lofts you desire, then click “set lofts no.” and put the number of the last node you want to be covered by each loft. For example, if you have a 19-node blade and 2 lofts, you might set the first loft to end at node 4 and the second loft to end at node 19. The default DTU10MW turbine blade has 194 nodes.

### 3.3.2 Outputs

Once the lofts are selected, click “Generate geometry/mesh.” BB3D will immediately generate a data.dat input file under models > DTU_10MW > Madsen2019 > BB3D . After a minute, it should generate a blade.igs CAD file in the same location. Note that BB3D does not have any error messages, so the only way to know whether it ran successfully is to see whether the blade.igs file was created.

## 3.4 PGL

Dependencies: PGL

PGL takes the text-based parameters and turns them into a mesh. These input parameters are taken from the default OpenTurbineCoDe blade at models > DTU_10MW > Madsen2019 > Madsen2019_10.yaml .

To run PGL, after loading the turbine case in the Main Options module, simply click the “Generate Geometry/Mesh” button in the Geometry module. The output mesh, in .xyz file format, can be found in models > DTU_10MW > Madsen2019 > PGL.

## 3.5 Salome

Dependencies: OpenCascade, BB3D, Salome

Salome takes the IGES file output by BB3D and turns it into a mesh that can be used with computational fluid dynamics software. Select “Salome” from the drop-down menu. If you already put the path to Salome into config.json, the “Get Salome location” box should be pre-filled; otherwise put the path to run_SALOME.exe into the box. Fill the “Load geometry” box with the path to the blade.igs file generated by BB3D. It should be under models > DTU_10MW > Madsen2019 > BB3D . You may choose whether to run in Debug Mode or not.

Click “Generate geometry/mesh.” If there are no errors, it will output a CGNS mesh file under models > DTU_10MW > Madsen2019 .

# 4. Aerodynamics Module

The aerodynamics module simulates the flow of air over turbine blades and outputs parameters such as thrust, torque, and coefficient of power.

## 4.1 AeroDyn (DLC = Uniform Flow)

Dependencies: AeroDyn Driver

The AeroDyn module is a standalone section of OpenFAST that takes text-based input files that describe wind turbine geometry and conditions, and it runs an aerodynamic simulation and generates output text files. The module runs very differently depending on which Design Load Case (DLC) is chosen, so this documentation splits it up into two sections. The current section deals with AeroDyn when the DLC is set to uniform flow.

### 4.1.1 Inputs

Most of the input files are pre-saved in the models > DTU_10MW > Madsen2019 > Aerodyn folder. The primary driver input file is DTU_10MW_ADdriver.inp — this contains general information about the turbine and the design load cases to be simulated. The AeroDyn input file is DTU_10MW_AD15.dat — this contains information about the tower and input and output files. The blade input file is DTU_10MW_ADBlade.dat — this contains information about blade geometry. OpenTurbineCoDe contains a default blade file, but if the AeroDyn part of the Geometry module is used, the blade file generated there overwrites the default one.

There are also several reference airfoil files located in models > DTU_10MW > Madsen2019 > Aerodyn > AeroData. These contribute to the geometry of the blade. For more information on AeroDyn input files, see the AeroDyn documentation: 4.2.1. AeroDyn Users Guide and Theory Manual — OpenFAST v3.2.0 documentation.

### 4.1.2 Instructions for Use

To run the AeroDyn module, first choose “Aerodyn” by the drop-down “Solver” box. Set DLC to “Uniform flow.” Next choose the desired values of fidelity, wind speed, tip speed ratio, pitch angle, and blade radius. Then click “Run simulation.” If multiple values are chosen for any of the parameters, AeroDyn will run multiple simulations (for example, the default setting lists four different values for both wind speed and tip speed ratio, causing AeroDyn to run four different simulations).

### 4.1.3 Outputs

An output file for each simulation can be found in models > DTU_10MW > Madsen2019 > Aerodyn > outputs. For information on interpreting each value in the output file, see this spreadsheet from the AeroDyn documentation: OutListParameters.xlsx (live.com).

Once simulations are run, the results can be plotted. The Aerodynamics module offers three types of graphs: thrust, torque, and coefficient of power plotted against the wind speed.

## 4.2 OpenFAST (AeroDyn with DLC ≠ Uniform Flow)

Dependencies: OpenFAST

If the DLC is not chosen as uniform flow, the AeroDyn module runs OpenFAST, an umbrella program which, for these purposes, includes AeroDyn, ElastoDyn, and InflowWind. AeroDyn simulates aerodynamic flow over blades; ElastoDyn simulates bending of solid bodies; and InflowWind simulates complex wind flow.

### 4.2.1 Inputs

Input files for all these programs can be found under models > DTU_10MW > Madsen2019 > OpenFAST. The primary input file is DTU_10MW.fst — this gives OpenFAST information about all the other input files. AeroDyn, ElastoDyn, and InflowWind each have their own input files that provide information to their respective segments of the program. AeroDyn has its own blade file; a default blade is provided by OpenTurbineCoDe, but if a blade was generated by the AeroDyn geometry module, this will replace the default blade file.

There are ElastoDyn input files for both the blades and tower, and AeroDyn reference airfoils are in the models > DTU_10MW > Madsen2019 > OpenFAST > AeroData folder. For more information about OpenFAST input files, see documentation of each module: 4. User Documentation — OpenFAST v3.2.0 documentation.

### 4.2.2 Instructions for Use

To run the OpenFAST module, first choose “Aerodyn” by the drop-down “Solver” box. Set DLC to 1.1 or 1.3 (The “internal” DLC option should take the design load case specified in the Main Options module, but functionality for this has not been developed yet, as of August 2022). Next choose the desired values of fidelity, tip speed ratio, pitch angle, and blade radius. Then click “Run simulation.” Because there are four default values for characteristic wind speed, four consecutive simulations will be run.

### 4.2.3 Outputs

An output file for each simulation can be found in models > DTU_10MW > Madsen2019 > OpenFAST > outputs. For information on interpreting each value in the output file, see this spreadsheet from the OpenFAST documentation: OutListParameters.xlsx (live.com).

Once simulations are run, results can be plotted. The Aerodynamics module offers three types of graphs: time-averaged values of thrust, torque, and coefficient of performance plotted against characteristic wind speed.

## 4.3 ADflow

Dependencies: ADflow

ADflow is a high-fidelity computational fluid dynamics solver that can be used to model airflow around wind turbine blades. It takes a CGNS grid input file and outputs CGNS mesh solution files for both surface and volume, as well as text-based output files. In OpenTurbineCoDe, it can analyze flow around the DTU_10MW reference turbine. For ADflow documentation, see here: https://mdolab-adflow.readthedocs-hosted.com/en/latest/index.html.

### 4.3.1 Inputs

ADflow input files can be found in models > DTU_10MW > Madsen2019 > ADflow > INPUT. The primary input files is DTU_10MW_L3.cgns, which is a grid file showing the geometry of the DTU10MW turbine and the surrounding air.

### 4.3.2 Instructions for Use

ADflow is called in OpenTurbineCoDe in a very similar way as AeroDyn. First choose “ADflow” by the drop-down “Solver” box. The “High fidelity mesh” box, which represents the resolution of the mesh must be set to 3, as looser meshes are not yet supported by OpenTurbineCoDe, as of August 2022. The DLC box must be set to “Uniform flow.”

Next choose the desired values of [DLC], wind speed, tip speed ratio, pitch angle, and blade radius. Then click “Run simulation.” If multiple values are chosen for any of the parameters, ADflow will run multiple simulations (for example, the default setting lists four different values for both wind speed and tip speed ratio, causing ADflow to run four different simulations). Given that ADflow is a high-fidelity tool, simulations may take several minutes.

### 4.3.3 Outputs

For each simulation, ADflow generates four output files, which can be found in models > DTU_10MW > Madsen2019 > ADflow > outputs. These output files are comprised of CGNS simulations for both the surface and volume flow, and text-based data files for lift distribution data and slice data.

Once simulations are run, results can be plotted. The Aerodynamics module offers three types of graphs: thrust, torque, and coefficient of performance plotted against wind speed.

# 5. Structures Module

The structures module simulates the forces that act on turbine blades and examines how the blades respond.

## 5.1 BeamDyn

Dependencies: BeamDyn Driver, PyFAST

The BeamDyn module is a standalone section of OpenFAST that takes text-based input files that describe turbine blade geometry and loads, and it runs a structural simulation and generates output text files. It can model either the DTU10MW turbine or a reference NREL5MW turbine.

### 5.1.1 Inputs

For the DTU10MW turbine, BeamDyn input files are pre-saved in the openturbinecode > structure > BeamDyn > DTU10MW folder. The primary driver input file is DTU10MW_driver.inp — this contains general information about the load cases to be simulated. The BeamDyn input file is DTU10MW_BeamDyn_Main.dat — this contains information about the simulation and geometry. The blade input file is DTU10MW_BeamDyn_Blade.dat — this contains more detailed information about blade geometry.

For the NREL5MW turbine, BeamDyn input files are pre-saved in the openturbinecode > structure > BeamDyn > NREL5MW folder. The primary driver input file is NREL5MW_driver.inp, the BeamDyn input file is bd_primary.inp, and blade input file is beam_props.inp. For more information about input files, see the BeamDyn documentation: 4.2.4. BeamDyn User Guide and Theory Manual — OpenFAST v3.2.0 documentation.

### 5.1.2 Instructions for Use

To run the BeamDyn module, select “Beamdyn” in the “Solver” box, select either the DTU10MW turbine or the NREL5MW turbine for analysis in the “Model” box, and press “Load.” The option to import a different User_Model turbine is not yet implemented in the code, as of August 2022.

BeamDyn offers three different options for the independent variable in its simulations: TipLoad x, DistrLoad x, and Twist Scale Factor. TipLoad x is the component of force at the blade tip along the X direction, measured in Newtons. DistrLoad x is the component of force along the X direction distributed across the length of the blade, measured in Newtons per meter. And Twist Scale Factor is a dimensionless coefficient that makes the blade more or less twisted.

To run a simulation, choose one of these three parameters and click the “Sweep” button next to it. (The other two parameters must have the “Cons[tant]” button selected so as not to interfere with the sweep). For the selected parameter, whether TipLoad, DistrLoad, or Twist Scale Factor, you must choose a minimum value, maximum value, and step size. BeamDyn will run one simulation for each step between minimum and maximum, including the minimum but not including the maximum.

For example, if TipLoad x is chosen as the independent sweep variable, 1 is chosen as the minimum, 2.5 chosen as the maximum, and 0.5 chosen as the step, BeamDyn will run three separate simulations. The first simulation will be at TipLoad x = 1 N, the second at TipLoad x = 1.5 N, and the third at TipLoad x = 2 N.

The optimization buttons and tab in the Structures module are not relevant to BeamDyn.

### 5.1.3 Outputs

Only one of the output files from the simulations is saved, either in openturbinecode > structure > BeamDyn > DTU10MW or in openturbinecode > structure > BeamDyn > NREL5MW, depending on which turbine was selected. The other output files are overwritten, though the critical data from them is saved in OpenTurbineCoDe. Output files track the following parameters in the x and y directions: force on the blade root in N (RootFxr, RootFyr), moment on the blade root in N-m (RootMxr, RootMyr), blade tip displacement in m (TipTDxr, TipTDyr), and blade tip angular displacement in radians (TipRDxr, TipRDyr).

Once simulations are run, results can be plotted. The Structures module can graph six different dependent variables against the chosen independent variable. These six dependent variables, which can be chosen in the “Response/objectives” are the maximum values of RootFxr, RootFyr, RootMxr, RootMyr, TipTDxr, and TipTDyr.

# 6. Control Co-Design Module

Dependencies: OpenFAST, PyFAST, WEIS

The Control Co-Design (CCD) Module can be used for two purposes. It can run simulations sweeping across different values of a control parameter in the Parametric Sweep submodule, or it can be used for design optimization in the Optimization submodule. As of August 2022, the CCD module is only functional when used with OpenFAST.

## 6.1 Parametric Sweep

As of August 2022, the Parametric Sweep submodule is only functional with the DTU 10MW reference turbine, though functionality should eventually be expanded to allow for user-generated turbine cases as well.

### 6.1.1 Inputs

The CCD module’s many input files are located mostly in the folder openturbinecode > controls > DTU10MWAero15. The main OpenFAST input file is DTU_10MW_NAUTILUS_GoM_A15_DLC1.2_Baseline.fst. Input files for submodules of OpenFAST generally have the submodule in the filename, end in .dat, and are also located in the DTU10MWAero15 folder (e.g. DTU_10MW_NAUTILUS_GoM_HydroDyn.dat for HydroDyn).

Of particular note is the OpenFAST ServoDyn submodule input file, which references the DISCON.IN controller input file, located in the DTU10MWAero15 folder. The DISCON.IN file itself references a file named Cp_Ct_Cq.DTU10MW.txt, which is located in the DTU10MWAero15 folder and contains performance tables for the power, thrust, and torque coefficients. The ServoDyn input file also references the libdiscon library — this is the ROSCO controller library, and it should be copied from the ROSCO build folder to the openturbinecode > controls folder (see installation documentation for details).

The input file for the OpenFAST InflowWind submodule has its own folder, DTU10MWAero15 > InflowWinds. This folder also contains a subfolder for design load condition input files. OTCD_DTU10MW.yaml

The CCD module also depends on the OTCD_DTU10MW.yaml file, which provides information about the reference turbine. It is located in the openturbinecode > controls folder.

### 6.1.2 Instructions for Use

In the top left corner of the CCD GUI, ensure that the “Model Selection” boxes read “OpenFAST” and “DTU10MW(Local).” Then click the “Load Models” button.

The CCD module offers three system parameters and five control parameters as options to sweep. To run a sweep, the “Sweep” checkbox should be checked next to the desired sweep variable, and all the other parameters should have “Cons” checkboxes checked. This will run simulations for values of the sweep parameter starting at the “Min” value and incrementing upwards by the “Step” value until (and not including) the “Max” value. For example, if the sweep parameter is chosen as “Omega in Region 2” with “Min” of 0.1, “Max” of 0.5, and “Step” of 0.1, the CCD module will run 4 simulations at Omega values of 0.1, 0.2, 0.3, and 0.4.

Parameters not chosen as sweep parameters will be set to their “Cons” values during these simulations.

What do these parameters mean? Tilt angle is the rotor shaft tilt angle, measured in degrees. Omega and zeta in region 2 are, respectively, the frequency (in rad/s) and the damping parameter of the pitch controller. Omega and zeta in region 3 are, respectively, the frequency (in rad/s) and the damping parameter of the torque controller.

It should be noted that as of August 2022, the parameters of Chord Scale Factor, Thickness Scale Factor, and Platform Feedback Gain are not properly implemented in the CCD module code, so useful parametric sweeps can be run only with the other five parameters.

Once a sweep parameter is chosen, choose a design load condition in the top right corner of the GUI. The options in the DLC box are uniform flow, 1.1, or 1.2 (the other options in the dropdown menu are not yet implemented in OpenTurbineCoDe, as of August 2022). If uniform flow is chosen, you can also specify an average wind velocity in the “Mean Velocity” box. For running sweeps with parameters in region 2, wind velocity should be <= 11.4 m/s, while for parameters in region 3, wind velocity should be > 11.4 m/s.

Finally, to run the parametric sweep, click on the “Parametric Sweep” tab in the bottom right section of the GUI. Paste the location of the OTCD_DTU10MW.yaml file into its box if it is not there already, and press the “Load” button to load the yaml file. Then press “Local Run” to run the sweep.

### 6.1.3 Outputs

Output files for the Parametric Sweep submodule are located in the same folder as the input files, openturbinecode > controls > DTU10MWAero15. The primary output file is DTU_10MW_NAUTILUS_GoM_A15_DLC1.2_Baseline.out. The CCD module overwrites its own output files as it runs simulations, so the only output file saved will be the one from the last simulation of the cycle.

Results of a sweep can also be graphed. To do this, first select the desired dependent variable — maximum torque (RotTorq_max) or maximum thrust (RotTorq_max) — in the “Response or objectives” box. Then click the “Plot” button.